Confined Space Program

[Name of Farm]

Disclaimer: The following Confined Space Safety written program is provided only as a template guide to assist employers and employees in developing plans which identify confined space areas and develop plans to limit the hazards associated with confined space areas. It is not intended to supersede the requirements of any OSHA standard. An employer will need to add information relevant to their particular facilities in order to develop an effective, comprehensive program.

CONFINED SPACE SAFETY PROGRAM

Name of Farm: ________________________________

I.	PURPOSE

This program will help to identify confined space hazards found on the farm. The purpose of this written Confined Space Safety program is to ensure that employers and employees know how to protect themselves from the hazards associated with entry into confined space work areas.  The program ensures that:

- Employers shall identify all areas which meet the OSHA definition of confined space. This is defined by OSHA Standard 29 CFR 1910.146.

- Employees will be made aware of all of the confined space locations on the farm and will be instructed not to enter these locations.

- Employees required to enter confined spaces as part of work duties will be trained in proper procedures for safely conducting work.

- The employer shall provide all necessary equipment to safely enter confined spaces.

Note:  This written program will be available to all employees for review upon their request and will be located in the following area:

Location:

II.	AUTHORITY AND REFERENCE

Occupational Safety and Health Administration (OSHA) General Industry Standards.

Safe Practices References:

ANSI/ASSE Z117.1-2009 Safety Requirements for Confined Spaces

OSHA 29 CFR 1910.146 Permit Required Confined Spaces

III.	DEFINITIONS

Permit Required Confined Space: 29 CFR 1910.146

Any space that:

- Is large enough and so configured that an employee can bodily enter and perform assigned work; and

- Has limited or restricted means for entry or exit (for example, tanks, vessels, silos, storage bins, hoppers, vaults, and pits are spaces that may have limited means of entry.); and

- Is not designed for continuous employee occupancy.

- A Permit-required confined space also has one or more of these characteristics:

  - Contains or has the potential to contain a hazardous atmosphere;

  - Contains a material with the potential to engulf someone who enters the space;

  - Has an internal configuration that might cause an entrant to be trapped or asphyxiated by inwardly converging walls or by a floor that slopes downward and tapers to a smaller cross section; and/or

  - Contains any other recognized serious safety or health hazards, such as unguarded machinery, exposed live wires, or heat stress.

Definitions:

Signage:  Visible, weather proof warning signs which identify confined space hazards. These signs should be in languages appropriate to the workers present at the location.

Awareness Training:  All employees of the farm are required to be trained annually in the location of confined spaces and that they should not enter confined spaces.

Entry Training:  Employees of the farm who are required to enter confined spaces to perform job functions shall be trained in the proper procedures for safely entering confined spaces.

Confined Space Plan: A written plan, specific to the individual farm, which identifies the confined space hazards on the farm.  The plan identifies who is allowed to enter confined spaces, documents trainings and procedures and lists required safety equipment.

Permit-required confined space: A confined space with one or more of the following characteristics:

- Contains or has the potential to contain a hazardous atmosphere.

- Contains a material that has the potential to engulf an entrant.

- Has walls that converge inward or floors that slope downward and taper into a smaller area that could trap or asphyxiate an entrant.

- Contains any other recognized safety or health hazard, such as unguarded machinery, exposed live wires, or heat stress.

IV.	APPLICATION

This program applies to all employees on the farm.

V.	RESPONSIBILITY FOR COMPLIANCE

A.	The administration of this program will be the responsibility of __________________ (person/position designated).  The administrative responsibilities of this individual/position will include:

1.	Identification of the confined spaces located on the farm and determining any ‘permit-required confined space’ as defined in section III.

2.	Inspection and upkeep of confined space safety equipment.

3.	Coordination and supervision of employee training, including classification of workers for awareness training and/or entry training.

4.	Coordination and supervision of the farm's confined space safety program.

6.	Coordination and supervision of required recordkeeping.

7.	Annual evaluation of the overall program.

B.	Employees are responsible for following all safe work practices and using proper precautions required by the guidelines in this program.

VI.	INSPECTION OF CONFINED SPACE HAZARDS

A.	_________________________ (person/position designated) is responsible for inspecting and evaluating all confined space hazards on the farm.

- All confined spaces shall have proper signage.

- All confined spaces shall have adequate restrictions that prevent entry into these areas.

- Inspect and maintain ventilation equipment

- Ensure appropriate rescue equipment is available and properly functioning.  Assess potential for hiring professional contractors to do repair and maintenance work in confined spaces.

- Eliminate entry into confined spaces whenever possible.

- Institute engineering controls to prevent exposures,

- Check to see if there are any changes in work, and amend the assessment and control measures accordingly.

- Determine if any confined spaces are ‘permit-required’.

B. The responsible person will inspect areas of the farm for the following confined space hazards:

1. Tank manure spreaders, manure storage and pumping areas.

2. Chemical storage areas.

3. Grain bins and other feed storage areas.

4. Milk bulk tanks.

5. Silos

C. _______________________ (person/position designated) is responsible for conducting annual inspections of confined space hazards.

D. ____________________ (person/position designated) must inspect hazards and make appropriate changes to meet the standards.

VII.	OPERATION INSTRUCTIONS

- The following will help minimize or eliminate the risk of confined space hazard exposures:

  - Post adequate and effective warning signage.

  - Conduct time of hire and annual trainings.

  - Create adequate barriers to entry.

  - Hire professional contractors to conduct confined space entry if possible.

  - Eliminate entry to confined space areas whenever possible.

  - Create safety protocols for all confined spaces determined to be ‘permit-required’ (see Forms)

VIII.	EMPLOYEE TRAINING

A.	Each employee will be trained in confined space hazards where they will receive information on the following topics:

- Recognize the hazards of confined spaces and to never enter without the proper training and equipment.

- Understand access limitations to specific confined spaces identified on the farm.

- Know safe rescue procedures and proper use of rescue equipment for employees that will be performing confined space entry.

- All training will be presented to employees in a manner that employees receiving it are capable of understanding (visual and oral presentations). Training presentations will be delivered in the primary language and at the appropriate literacy levels of the workers.  During, and at the end of training, an assessment will be made to see that employees understood the training.

C.	Upon completion of the training program, each employee will sign a form documenting that he/she has received the training. (See Form #2)

D.	Whenever a new employee is transferred or hired, he/she will be provided training regarding confined space safety if it will be part of their assignment.  The training session will be conducted by ________________ (person/position designated) before the start of his/her employment.

E.	___________________ (person/position designated) will conduct training on the specific hazards of the job and the appropriate personal protective equipment and safety precautions and procedures.

F.  Refresher training will be done by ___________________ (person/position designated).  Refresher training can be delivered for the following circumstances; when job duties change, after an incident, or close call or when supervisor inspections reveal improper procedure. Refresher training will follow the procedures outlined in IX (A-D).

IX.	INFORMATION TO CONTRACTORS

A.	___________________________ (person/position designated) is responsible for providing outside contractors with any and all information regarding confined spaces potential hazards and any permit entry requirements.

X.	PERSONNEL POLICIES

When an employee is not following safety rules regarding confined space hazards, disciplinary action will be taken.

LIST OF SAMPLE FORMS

Form 1		CONFINED SPACE SAFETY OPERATIONS SUMMARY

Form 2		EMPLOYEE CONFINED SPACE SAFETY TRAINING RECORD

Form 3		CONFINED SPACE ASSESSMENT FORM

Form 4		PERMIT-CONFINED SPACE ENTRY

Form 5		CONFINED SPACE FLOW CHART

Form #1

CONFINED SPACE SAFETY OPERATION ANNUAL

PROGRAM SUMMARY

Training

|  | Number of Training Courses Presented: | Number of Employees Trained: |
| --- | --- | --- |
| New-employee training: |  |  |
| Facility-specific training: |  |  |
| New-facility training: |  |  |
| Other training: |  |  |
| Total courses/employees |  |  |
| Confined Space Inspections |  |  |
|  | Number of facilities with confined space | Hazards inspection completed |
| Previous Total: |  |  |
| New This Year: |  |  |
| Revised Total: |  |  |

The following activities have been completed:

Written program is up to date.

Servicing and maintenance protocols are up to date.

All training is up to date.

All confined space hazards have been inspected

If any of the above activities are not complete, explain:

Completed By: __________________________________________ Date: __________________

Form #2	          EMPLOYEE CONFINED SPACE SAFETY TRAINING RECORD

The following employee(s) have completed training in Noise Safety.  Each trained employee is now knowledgeable in all 4 different training topics covered in the Noise Safety Training.

- Recognize the hazards of confined spaces.

- Understand access limitations to specific confined spaces identified on the farm.

- Know safe rescue procedures and proper use of rescue equipment

| Employee's Name | Employee's Signature | Date of Training | Trainer | Trainer's Signature |
| --- | --- | --- | --- | --- |

Form #3			Confined Space Assessment Form

Farm Location

| Confined Space Location: |  |  | Purpose of Entry: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Size (Cu. Ft.): |  |  | *Please attach photo of space to this form |  |  |  |  |  |
| To determine if the space listed above is a confined space, read each of the following statements and CIRCLE the response that best applies to the space being evaluated. |  |  |  |  |  |  |  |  |
| The space is large enough and is so configured that an employee can bodily enter and perform assigned work. |  |  |  |  |  |  | YES | NO |
| The space has limited or restricted means of entry or exit. (i.e. tanks, vaults, vessels, silos, storage bins, hoppers, pits, diked areas) |  |  |  |  |  |  | YES | NO |
| The space is not designed for continuous employee occupancy. |  |  |  |  |  |  | YES | NO |
| If all three of the above statements were answered yes, the area being evaluated is a confined space. Continue to the next section to determine if it is a “permit-required” confined space. If any of the above statements were answered no, proceed no further. The area does not qualify as a “confined space.” |  |  |  |  |  |  |  |  |
| Use the following section to determine space is “permit-required”. |  |  |  |  |  |  |  |  |
| 1. The space contains, or has the potential to contain a “hazardous atmosphere”. CIRCLE yes or no, and check all those that apply. Specify hazard when possible on back. |  |  |  |  |  |  | YES | NO |
| __ ___ Flammable gas / vapor / mist |  |  |  | ____ Oxygen conc. below 19.5% or above 23.5% (Test oxygen levels with gas meter) |  |  |  |  |
| _____ Airborne combustible dust conc. > or = its LFL. Dust obscures vision @ 5 feet or less. |  |  |  | _____ Atmospheric concentration exceeding the PEL or dose for any substance published in subpart G or Z. |  |  |  |  |
| _____ Any other atmospheric condition that is IDLH. (i.e. poor ventilation, migrating vapors / gases) |  |  |  | _____ Other: (inerting gases, etc) List: _____________ _______________ |  |  |  |  |

| 2. The space contains a material that has the potential for engulfing an entrant (i.e. fill or plug respiratory tract, cause death by crushing, constriction or strangulation) |  |  |  |  | YES | NO |
| --- | --- | --- | --- | --- | --- | --- |
| 3. The space has an internal configuration such that an entrant could become trapped or asphyxiated. CIRCLE yes or no, and check all those that apply. |  |  |  |  | YES | NO |
| _____ Converging walls / downward sloping floors |  |  | _____ Constriction / taper to a smaller cross-section |  |  |  |
| _____ Difficult to exit / inadequate access / obstacles |  |  | _____ Other: List: ________ | ____________ |  |  |
| 4. The space contains other recognized serious safety or health hazards. CIRCLE yes or no, then check all those that apply. *Specify hazard on the back. |  |  |  |  | YES | NO |
| _____ Poor or difficult communication |  |  | _____ Noise / vibration |  |  |  |
| __ __ Equipment startup / mechanical hazard |  |  | _____ Inadequate light / poor visibility |  |  |  |
| _____ Hot or cold contact / extremes |  |  | _____ Slip & trip surfaces, fall from heights |  |  |  |
| _____ Electrical shock | Water? | Lights | _____ Sharp objects / falling objects |  |  |  |
| _____ Release of stored & hazardous energy |  |  | _____ Unknown contents / sludge / residue |  |  |  |
| _____ Flooding / avalanche of materials / engulfment |  |  | _____ Radiation |  |  |  |
| _____ Toxic chemicals |  |  | _____ Atmospheric contaminant |  |  |  |
| _____ Corrosive materials |  |  | _____ Chemical reactivity |  |  |  |

# Environmental/Occupational Health & Safety Management Program

Confined Space Assessment

Please attach a photo or photos of this space for reference.

Answer all questions as completely and to the best of your ability. Attach additional paper if you need more space.

- Please list details of exiting or potential atmospheric hazards a (#1 on front page of Assessment Form):

- Please list details of exiting or potential engulfment hazard (#2 on front page of Assessment Form):

- Please list details of exiting or potential entrapment hazard (#3 on front page of Assessment Form):

- Please list details of exiting or potential recognized serious safety or health hazards (#4 on front page of Assessment Form):

- How many times is this space entered annually?   Zero     1-3      4-6    7-9    10+

- Which employees are required to enter this space?

- Can hazards be minimized or removed?                Yes          No

If yes, how can this be accomplished?

Adapted from the UW-Madison Environmental Safety and Health Confined Space Assessment Program, July 2009.

Form #4			PERMIT-CONFINED SPACE ENTRY

Confined Space Location/Description/ID Number				Date:

______________________________________________________________________________________

Purpose of Entry

____________________________________________________________________________________________________________________________________________________________________________

Time In: ______________	Permit Canceled Time: _____________________________________

Time Out: ____________		Reason Permit Canceled: ___________________________________

Supervisor: ___________________________________________________________________________

Rescue and Emergency Services-

| Hazards of Confined Space | Yes | No | Special Requirements | Yes | No |
| --- | --- | --- | --- | --- | --- |
| Oxygen deficiency |  |  | Hot Work Permit Required |  |  |
| Combustible gas/vapor |  |  | Lockout/Tagout |  |  |
| Combustible dust |  |  | Lines broken, capped, or blanked |  |  |
| Carbon Monoxide |  |  | Purge-flush and vent |  |  |
| Hydrogen Sulfide |  |  | Secure Area-Post and Flag |  |  |
| Toxic gas/vapor |  |  | Ventilation |  |  |
| Toxic fumes |  |  | Other- List: |  |  |
| Skin- chemical hazards |  |  | Special Equipment |  |  |
| Electrical hazard |  |  | Breathing apparatus- respirator |  |  |
| Mechanical hazard |  |  | Escape harness required |  |  |
| Engulfment hazard |  |  | Tripod emergency escape unit |  |  |
| Entrapment hazard |  |  | Lifelines |  |  |
| Thermal hazard |  |  | Lighting (explosive proof/low voltage) |  |  |
| Slip or fall hazard |  |  | PPE- goggles, gloves, clothing, etc. |  |  |
|  |  |  | Fire Extinguisher |  |  |

Communication Procedures:

| DO NOT ENTER IF PERMISSABLE ENTRY LEVELS ARE EXCEEDED |  | Test Start and Stop Time: Start Stop |  |
| --- | --- | --- | --- |
|  | Permissable Entry Level |  |  |
| % of Oxygen | 19.5 % to 23.5 % |  |  |
| % of LEL | Less than 10% |  |  |
| Carbon Monoxide | 35 PPM (8 hr.) |  |  |
| Hydrogen Sulfide | 10 PPM (8 hr.) |  |  |
| Other |  |  |  |

Name(s) or Person(s) testing: ________________________________________________

Test Instrument(s) used- Include Name, Model, Serial Number and Date Last Calibrated:

| CFM-Ventilation | Size-Cubic Feet | Pre Entry Time | - Central Notified Before Entrance | Time Notified: |
| --- | --- | --- | --- | --- |
|  |  |  | - Central Notified After Entrance | Time Notified: |

Authorized Entrants					Authorized Attendants

________________________________		_____________________________

________________________________		_____________________________

| PERMIT AUTHORIZATION I Certify that all actions and conditions necessary for safe entry have been performed. |  |
| --- | --- |
| Name-Print: |  |
| Signature: |  |
| Date: | Time: |

Entry Procedure Checklist: Complete the following steps before, during, and after a confined space entry:

## Step 1

Obtain a Permit-Confined Space Entry Form from Program Coordinator.

## Step 2

Notify Supervisor before the Confined Space Entry

## Step 3

### Verify Confined Space Meter has been calibrated and is in working order

## Step 4

### Complete the top portion of the Permit-Confined Space Entry Form

## Step 5

Ensure all rescue equipment (e.g. tripod, body-belt, lanyard) is in place prior to entry

## Step 6

The confined space must be continually ventilated with intrinsically safe ventilation equipment

## Step 7

### Monitor the confined space with a four gas meter prior to entry.  The entrant and attendant should sign the permit authorization section on the bottom of the permit to ensure all actions and conditions necessary for safe entry have been performed.

## Step 8

Employee entering the confined space should wear four gas meter after the pre-atmosphere test.  The employee should also have a full body harness and lanyard attached to the rescue tripod.

## Step 9

### Employee can enter the confined space once Step 8 is completed.  The entrant and attendant should complete the Hazards of Confined Spaces and Special Requirements Section of the Permit-Confined Space Entry Form once the employee is within the confined space.  The entrant should also gather the % Oxygen, % Explosive Gases, Carbon Monoxide, and Hydrogen Sulfide readings and communicate them to the attendant to place on the Permit Form.

Step 10

The attendant should maintain constant communication with the entrant until the entrant has exited the confined space.

Step 11

The attendant should contact Supervisor once the entrant has exited the confined space.

Step 12

The Permit-Confined Space Entry Form should be given to program coordinator, to file in the Confined Space Records.